PRELIMINARY DRAFT
(October 2, 2007)

ME72: Engineering Design Laboratory
“FIRE and FLY”
Contest Description and Rules
Twenty-third Annual Engineering Design Competition

Tuesday, March 11, 2008
1. Introduction

The contest this year involves the construction of:

(1) a projectile or flying device that will rise from a ground-level launcher, fly over a rope hurdle and then glide or be projected as far as possible over clear ground. This device must carry a 100 gram payload (likely to consist of a stack of metal washers).  Each team will be provided with at least one such payload. 
(2) a ground-level launcher device that will help propel the flying/projectile device on its way. 

The final contest will take place on Tuesday, March 11, 2008 in the Court of Man on the Caltech campus.  The launchers can be placed at any location within a 1 meter x 1 meter square launch area which will be positioned close to the front of the Beckman Auditorium.  Each team (consisting of two people) will have 45 seconds to place their system within the launching zone, and then a 15 second period to actuate/release their system.  The rope hurdle will be strung between the Beckman Behavioral Biology building and Baxter Hall.  The rope will not be strictly horizontal, but will sag such that its minimum elevation of 5 meters will be located near the mid-point between the two buildings. The rope will be placed approximately 30 meters from the launch zone. The measure of success will be the distance that the payload is transported in a southerly direction from the rope hurdle. The final resting place of payload washer must lie between two boundary lines (with a north-south orientation) that will be placed on each side of the Court of Man and extending into the space between Dabney Hall and the Parsons-Gates building. If the payload ends up outside the boundary lines, then the landing location will be marked as the point where the flying device last crossed a boundary line, as estimated by an official line judge. Special admiration and extra bonus, in terms of a distance added to your actual distance score, will be reserved for those payloads that make it to a prescribed circular zone (with a 2 meter radius) at the front of the Parsons-Gates building.  

As described in more detail below, each team will have the potential ability to control their flying/projectile device during the launching and air-borne periods via a standard hobbyist remote control system.  A fixed number of servo motors, battery packs, and a radio control receiver will be provided to each team so that they can be engineered into their design solution.  During the contest, at least one team member must be initially positioned within a 3m x 3m box surrounding the launch area.  The second team member can be initially positioned at any location north of the rope hurdle, and between the lateral contest boundaries. After the launch, both team members may move as may be needed to monitor and control their device.  However, at no time can a team member, or any other person, have contact with either the launching or flying components of their system until the payload has come to rest.  Moreover, no team member or other person may interfere with the movement or activities of their competitors during the setup, launch, and flight periods.
2. Energy Sources

Every team will be provided with the same energy sources and actuators, which will consist of 
1) Possible potential energy at higher ground level locations within the launch area.

2) A small compressed CO2 cartridge. These cartridges will be provided free but it is not permissible to inject gas into these cartridges or heat them up in a way that would increase the compressed gas energy within. It is also not permissible to remove material from the cartridge in order to decrease its mass.

3) A “small” battery pack for the radio-controlled receiver and hobbyist servo-motors (described below).

4) A large battery pack and two D.C. servomotors that are compatible with the battery pack.  Neither the large battery pack nor the D.C. servomotors can leave the launch area from the time in which the launching sequence is started until the payload comes to rest.
5) Energy stored in a spring-like device made from a limited and prescribed amount of standard contest materials (see below).
6) Note: it is not permitted to use a propeller, or any device that can be construed as a propeller, on the flying/projectile device.

Each team will be have access to a standard model-airplane radio-controller with at least 4
channels, and  hobbyist servomotors (two small, and two medium sized). These cannot be modified in any way; they must be used as they are provided to you.  Similarly, the battery packs provided can not be chemically or thermally altered. 
3. Launching
· Prior to each launch, the design team will be permitted a maximum set up time of 45 seconds (beginning with the instructors’ start command) to place their device(s) within their launching zone.  The launching zone will consist of a simple flat slab into which four ½ inch holes have been tapped.  Contestants may choose to (but are not required to) use these mounting holes to fasten and stabilize their device during the launching period.
· Two launching pads will be available for each competition.  They will be placed 3 meters apart, symmetrically located about the midline of the competition zone.
· At the end of the 45 second placement period, the total design system must be statically stable, prior to launch activation.  Moreover, at the end of this 45 second placement period, each contest entry must fit within an imaginary rectangular box that is 80cm x 80cm at the base, and 2m in height.
· Each system must have a launching mechanism that can be simply activated by the press of a switch, or an analogous device.  A switch will be provided in the contest material list. Prior to activation of the launching switch, the launcher and the flying device must all be static and must be contained within the imaginary box described above.
· The launch sequence may take up to 15 seconds.  Starting from the instant at which the launch switch is depressed or activated, the contestants (or any other person) may have no further physical contact with their device, except for purposes of dismantling their device and removing it from the launch area.  However, internal mechanisms within the launching system may carry out tasks, and remote control of the launching system and flying/projectile component vias the radio controller system and servo motors is allowed during the launching period. However, if no portion of the design system has left the imaginary box by the end of the 15 second period, then that contest entry will be assigned a zero travel distance.
· No external electrical connection to the launcher or to the on-off switch will be permitted.  The switch may only be connected to the battery packs, servo motors, and controllers provided to each team.
· After launching, the launching subsystem (and all of its parts which are clearly not part of the flying/projectile component) must come to rest within a circle of radius 130cm whose center is the center of the imaginary box described above.
· After the end of the launching period, all parts of the system must be removed from the launching zone within 45 seconds.  
4. Evaluation
An overall winner will be determined in a double elimination tournament. This is just like a single elimination tournament, except a team must lose twice (instead of once) to be eliminated from the competition. Pairing of teams to compete will be chosen randomly at the start of each round of the tournament. The team selected first in the random draw will go first in every contest. In each contest pairing, the team whose payload comes to rest furthest south, according to the rules described above, will be declared the winner. Again, recall that a flying/projectile device that lands outside of the proscribed boundaries will have its southerly progress marked at the point where the device crossed the boundary for the last time. Each boundary will be staffed by a line judge who will determine this point. In all cases (particularly those requiring judgment) the judges will decide the winner.

5. Individual Work
While you will be working in teams of two, it is expected that each individual will design and fabricate a functional device. It is acknowledged that interaction between teams in the class is highly beneficial. To that end, any conversations, calculations, analyses, ideas and tests may be shared among the teams, but the device design and fabrication must be an individual team effort. Note that this collaboration policy does not extend to replicating others’ ideas. Occasionally two teams will arrive independently at very similar solutions.  Sometimes one team will see a great idea in another team’s device, and finding no superior alternative, will want to incorporate it. This duplication is permissible, however, not encouraged. In the past, some competitors have maintained a high level of secrecy around their device, and blindly copying an idea or strategy may be risky. In many respects, you should treat this design project as similar to an ordinary homework set. It is permissible to collaborate with your classmates and seek the advice of the instructor, TA’s, M.E. Shop staff, other class participants, and other students.  However, the final product must be your own work. At the end of the term, for grading purposes, you must be able to indicate the boundary of the functional device that you designed and fabricated, and in particular to distinguish it from the parts of the device that your teammate designed and fabricated. Furthermore, it is vital to document both your own work and the contributions of others to your ideas by detailing the process of ideation, design, and fabrication in your design notebook. If you are concerned about the acceptable limits to collaboration, discuss the situation with the instructor(s). Do your own work, and as always, it is best if you use your own ideas and concepts.

6. Additional Clarifications of the Contest Rules
6.1. Safety:

(a) Any device which is judged to have the potential to cause injury to any participant or spectator will be disqualified.

(b) It is mandatory that safety glasses be worn at all times while competing and testing. This requirement will be relaxed during the final tournament.  It is, of course, also mandatory that safety glasses be worn at all times while in the M.E. Shop.

6.2. Energy Sources:

The energy used by each contestant’s device is limited to the following:

(a) Electrical power supplied by a “small” and a “large” battery pack that is provided to each design team. Students will have access to an ample number of battery chargers throughout the course and during the contest.  However, design teams may choose to purchase additional battery packs if so desired in order to ensure maximum charge during the contest.  But no more than one small and one large battery pack may be used at any one time in the system design and deployment. Additionally, batteries will be provided during the testing and contest period for the hand-held radio control modules. The battery packs may not be modified in any way at any time (mechanically, chemically, thermally, electrically, etc.); they must be stored and operated at ambient temperature. Used batteries must be recycled when depleted of electrical energy. A container for this purpose will be available in the M.E. Shop.

(b) There will be limits on the spring energy stored by deforming any element or elements of the “bag of junk”, including the springs and rubber bands. The limit of the spring energy that can be stored in the launcher will be 20.0 joules (approximately the amount of spring energy stored by deforming six (6) medium sized rubber bands supplied in the “bag of junk”); the material in which this energy is stored must remain in the launcher throughout the duration of a contest round.  The limit on the spring energy that can be stored in the flying device will be 10.0 joules (approximately the amount of spring energy stored by deforming three (3) medium sized rubber bands). Testing and analysis that demonstrates conclusively that these limits have been met must be included in your design notebook, and approval of your analysis must be indicated by the signature of the instructor. 

(d)  The energy contained in a single CO2 cartridge supplied by the course instructors.

Replacement cartridges will be available. The first replacement cartridge will be supplied upon request. All subsequent replacement cartridges will only be provided at the time a used cartridge is turned in. Each contestant will be limited to a maximum of eight (8) cartridges.  Additional cartridges can be purchased by each team if desired, but at no time can a team’s contest entry contain more than one cartridge.
6.3. Contest Materials:

Each contestant’s device(s) must be constructed entirely from materials supplied in the contest materials list (which will be provided in a separate handout).  No other materials (either from the M.E. Shop, or elsewhere) can be used or substituted, with exceptions described immediately below.  In addition to the standard material list each contestant may also use:

(a) A maximum of 113.4 grams [4.0 ounces] (dry cured mass) of RTV silicone casting compound. This casting compound is supplied in bulk. See the Staff in the M.E. Shop if you wish to cast one or more parts out of silicone. The intended purpose of this casting compound is for sealing, however, it may be used for molding components.

(b) A maximum of 3 meters [118.1 inches] of 2.38 mm [3/32 inch] diameter “Orange-Go” belt material may be used. The M.E. Shop staff can show you how to join the ends of a segment of this material into a continuous belt. The intended purpose of the “Orange-Go” is for power transmission as a belt, however, it may be used as a tire or track material, or for other (non-decorative) purposes.

(c) Glues and epoxies that are used only for bonding. Among the prohibited uses of glues is the creation of a composite material using glue as the matrix.
(d)  Paint or Shellac may be used to insulate strands of wire, if desired.  Similarly, up to one meter of black electrical tape may also be used for purposes of electrical insulation.  Such tape may not be used for structural purposes.
(e) Light machine oil, mineral oil, or vegetable oil (depending on the competitor’s preference) can be used SPARINGLY to lubricate moving parts.
(f) Non-functional decorations (such as decals and paint) can be used, and are encouraged. 

 Each contestant’s final device(s) must be able to have been fabricated from the materials or parts contained in the parts list, with the exceptions described above.

Replacement supplies and materials are available on a limited basis. If you damage something, or cut it up and then want to do something different with it, see the M.E. Shop staff or one of the TA’s. We will do our best to supply replacements, but we cannot guarantee unlimited supplies of all materials. We also cannot guarantee that replacements will be identical to the original. You may want to check availability of replacements prior to conducting a risky experiment with a rare part.

Contestants are responsible for providing their own glues and epoxies. Some glues and epoxies will be available in the M.E. Shop, but to ensure an un-interrupted supply, go to a (hardware) store, and buy your own.
The approved contest materials may be mechanically modified in any way (disassembled, cut, machined, turned, ground, etc.). However, they may not be altered chemically (except locally by glues, for bonding, or paint for decoration or insulation purposes).

Soldering and brazing are permitted, though not particularly encourage. Welding is not permitted.

6.4. Contest Device:

(a) Size:

i. Prior to the start of the tournament, each team must demonstrate to the course instructors that, in its immediate prelaunch configuration, all the devices will fit into an imaginary 80 x 80 x 200 cm box.  A measuring device will be available in the M.E. Shop to check this constraint.

ii. At the start of the each contest (t=0), each team’s combined devices must be positioned within the proscribed launch area as described above.
(b) Mass: Each team’s total design must not weigh more than 7.5 kg, including the mass of the supplied actuators and battery packs (but not including the hand-held radio control module or the 100 g payload).

(c) Finalized Design. After each device’s initial flight during the final contest, no major design changes or construction will be allowed. Crashes of the flying devices are inevitable and repairs to these devices using materials from the approved materials list will be permitted provided that, in the judges’ opinion, they involve no major design changes to the initial device. However, changes to flight parameters such as angles of incidence or rebalancing of the flying device will not only be permitted but are encouraged.  Any such minor corrections to the device must respect the rules on size, mass, and materials described elsewhere.
(d) Interaction with the device.  Other than radio control through the servomotors and the radio transmitter, and through the activation of the launching switch or device, no manipulation of, or interaction with, a device will be allowed while it is competing (i.e., after the start of the launching period, up to the time that the payload touches the ground).
(f) Team Number.   Each team must choose a 2-digit integer number to identify their team and their contest device(s).   Each major component of each team’s design must carry an identifying team number.  Note that Number 00 is reserved for the contest placebo. Check with the instructor(s) to avoid duplicate numbers.  While not required, teams are also encouraged to choose a team name.
6.5. Spatial Rules:

(a) The contest arena includes all air-space.
(b) Each device must be designed to functionally interact with nothing other than the launch area,   the air, the ground, and the remote control system.  Bouncing off buildings and other exotic strategies that deploy the buildings, trees, or other infrastructure in the vicinity of the contest are not allowed.
(c) At the start, before the launch switch is activated, all portions of each team’s devices must rest entirely within that team’s start volume, as defined by the imaginary box above.
(d) You may not interfere in any way with the conduct of any opponent’s device or with their flight. Thus you may not interact with, modify, or intentionally block the radio control signals from the opposing teams’ transmitter.

(e) Intentional damage to any receiver/control module will result in disqualification. It is your responsibility to avoid damage to the receiver/control module, both during testing, and during the contest. Competitors should make efforts in the design and fabrication of their devices to avoid placement of the receiver/control module in a vulnerable position.
6.6. Contest Arena:

(a) The final contest will take place in the Court of Man. The launchers can be placed at any location within a defined launch area which will be close to the front of Beckman Auditorium. The rope hurdle will be strung between the Beckman Behavioral Biology building and Baxter Hall. It will not be horizontal but will sag such that its minimum elevation of 5 meters will occur near the mid-point between the two buildings.  The rope hurdle will be placed 30 meters south from the launch area.  The rope will be oriented in as nearly an east-west direction as is possible.
(b) Boundary lines that are approximately north/south will be marked on each side of the Court of Man and will extend into the space between Dabney Hall and the Parsons-Gates building. Note that the boundary lines may not necessarily be straight.  The boundaries are implied to extend vertically upward from these lines placed on the ground. As stated earlier, the final resting place that will determine the winner of each contest will be the southerly location of the payload, provided it comes to rest inside the boundary lines. If the payload ends up outside one of the boundary lines then the final resting place will be the location where the payload last crosses the boundary, as judged by the line judge responsible for that boundary.  Additional points (in the form of a distance added to a team’s flight distance) will be awarded to entries that land in a circle of 2m radius placed in front of Parson’s Gates.
(c) Prior to the final contest, participants are encouraged to use the Court of Man for their practice flights. However no overhead rope, no launch area, and no boundary lines will be provided prior to the contest day for such practices. 
(d) Any intentional damage to Caltech buildings, grounds, or other infrastructure, other than that within the launch area, will result in disqualification.

6.7. Time:

(a) During the tournament, contestants will be called up “On-Deck” while the preceding flight is taking place and should be prepared to begin their set-up period of 45 seconds as soon as the preceding flight is concluded and the preceding contestants clear the launching zone of their entry.
(b) Exceeding the 45 second set-up time will result in a loss in that round for the offending team. 
(c) After the end of the set-up time, no action of a team’s device(s) is permitted prior to the initiation of the launch switch.  I.e., during the setup time, it is not permissible for motors to be working to store energy.  Only those movements of the team’s mechanisms that are needed to place the devices in the launch region, and to bring the devices to rest by the end of the 45 second set-up period are allowed.  
(d) During the 15 second launch period, which must be initiated by an electrical switch or analogously simple mechanism, any movement of the mechanism (as long as it is unaided by direct human contact) is allowed.  The flying/projectile portion of the device must leave the launch region by the end of the 15 seconds.  If no portion of the contestant’s device(s) leave the launch region after 15 seconds, a flight distance of zero meters will be assigned to the team for that round of the competition.  

(e) A maximum pick-up time of 45 seconds will be allowed for removing all of your device(s) from the arena after a flight.

6.8. Tournament:

(a) Teams will compete in a double-elimination tournament to determine the wining team.

(b) Teams will be chosen at random (during the tournament) to compete against each other in each round.

(c)  If, after any flight in which the flying device fails to fly over the suspended rope, the payload within the flying device does not end up outside the launch area, that effort will be counted as a loss irrespective of the performance of the competitor. 

(d) If the flying device of neither team manages to fly over the suspended rope then the team whose flying device ends up furthest south will be declared the winner (the rules pertaining to devices that end up outside the north-south boundaries will be as described previously) provided the payload of that device ends up outside the launch area.

(e) If only one of the two flying/projectile devices manages to fly over the suspended rope, then that team will be declared the winner irrespective of the location of its final resting place.
(f) During a contest, if your team gets a bye, or your opponent(s) do not show, your team must be prepared to compete against a placebo device.

6.9. Miscellaneous:

7. Weather

Since the outdoor contest will be held in March, there is a possibility that rain may cause disruption of the event. It is likely that we will proceed with the contest in the event of light or occasional rain showers.  Otherwise the event may be moved to the gym which is available on that afternoon. However, some of the parameters of the contest may have to be changed to fit within the confines of the gym.  Alternatively, the contest may be postponed.
